ERP Requirement Template

TNO			: GGA0373

USER NAME		: SARAVANESHWARAN.C

PROCESS FLOW	: LINE TABLE & SEWING MACHINE ELECTRICAL PREVENTIVE MAINTENANACE ENTRY THROUGH QR CODE SCAN

MODULE NAME	: ELECTRICAL PREVENTIVE MAINTENANACE ENTRY THROUGH QR CODE SCAN

COMPONENT NAME	:

MENU LINK

SCREEN

DESCRIPTION:

1. ELECTRICAL CHECK POINTS FOR SEWING MACHINE PREVENTIVE MAINTENANCE ENTRY – MONTHLY ONCE WISE

2. ELECTRICAL CHECK POINTS FOR LINE TABLE PREVENTIVE MAINTENANCE ENTRY – MONTHLY ONCE WISE

CONDITIONS:

| CONDITIONS |  |  |
| --- | --- | --- |
| 1 | VERY MACHINE MAINTANANCE TAKEN FOR 1 MONTH ONCE IN BETWEEN QR CODE FOR PREVENTIVE MAINTENANCE IT WILL SHOWS | ALREADY COMPLETED |
| 2 | YELLOW COLOUR AND BOLDED HIGHLIGHTED POINTS | AFTER ENTERED THE VALUES ONLY SUBMISSION WILL DONE |
| 3 | REPORT START TIME AND END TIME SHOULD NEEDED | AFTER SACNNED TIMER SHOULD START AND AFTER SUBMISSION DONE TIME SHOULD STOP |

JOURNAL ENTRY (IF APPLIED): - CHECK POINTS DETAILS

| SEWING MACHINE PREVENTIVE MAINTENANCE - MONTHLY ONCE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| SL.NO | CHECK POINTS | VALUES | UOM | EXAMPLES | UOM |
| 1 | CONNECTOR CHECKING | YES / NO |  | YES / NO |  |
| 2 | VOLTAGE CHECKING | ALPHA NUMERIC | VOLTAGE | 230 | VOLTAGE |
| 3 | LOAD AMPS CHECKING | ALPHA NUMERIC | AMPS | 1.25 | AMPS |
| 4 | BOARD CHECKING | YES/NO |  | YES / NO |  |
| 5 | MOTOR CHECKING | YES/NO |  | YES / NO |  |
| 6 | MACHINE LED FITTING | YES/NO |  | YES / NO |  |
| 7 | AIR LEAKAGE SAVING CFM | ALPHA NUMERIC | CFM | 0.5 | CFM |
| 8 | CLEANING WORK | YES/NO |  | YES / NO |  |
| 9 | REMARKS | ALPHA NUMERIC |  | VALUES OR NOS |  |
| LINE TABLE PREVENTIVE MAINTENANCE - MONTHLY ONCE |  |  |  |  |  |
| SL.NO | CHECK POINTS | VALUES | UOM | EXAMPLES | UOM |
| 1 | BUS BAR CHECKING | YES/NO |  | YES/NO |  |
| 2 | PLUG TOP CHECKING | YES/NO |  | YES/NO |  |
| 3 | AIR LEAKAGE SAVING CFM | ALPHA NUMERIC | CFM | 0.5 | CFM |
| 4 | TUBE LIGHT CHECKING | YES/NO |  | YES/NO |  |
| 5 | NUMBER OF MACHINE RUNNING | ALPHA NUMERIC | NOS | 15 |  |
| 6 | LOAD AMPS CHECKING | ALPHA NUMERIC | AMPS | 0.5 | AMPS |
| 7 | CLEANING WORK | YES/NO |  | YES/NO |  |
| 8 | CONNECTOR CHECKING | YES/NO |  | YES/NO |  |
| 9 | REMARKS | ALPHA NUMERIC |  | VALUES OR NOS |  |

NARRATION (IF APPLIED):

ENTRY FLOW

APPROVAL SCREEN REQUIRED?

YES

APPROVED BY:

2 LEVEL APPROVEL NEED FOR THAT

ENTRY BY – ELECTRICIAN.

FIRST LEVEL APPROVEL BY – ELECTRICAL SUPERVISOR.

SECOND LEVEL APPROVEL BY – ELECTRICAL MANAGER.

APPROVAL MENU LINK:

REPORT REQUIRED?

YES

TYPE OF REPORT:

POWER BI

REPORT FORMAT:

| REPORT FORMAT IN PBI |  |  |  |
| --- | --- | --- | --- |
| SEWING MACHINE PREVENTIVE MAINTENANCE REPORT |  |  |  |
| UNIT | GARMENTS 1&2 FLOOR | START TIME | 12:30 PM |
| MACHINE NO | O/L-413 | END TIME | 12:45 PM |
|  |  | TOTAL TIME | 00:15 |
| SL.NO | CHECK POINTS | EXAMPLES | UOM |
| 1 | CONNECTOR CHECKING | YES / NO |  |
| 2 | VOLTAGE CHECKING | 230 | VOLTAGE |
| 3 | LOAD AMPS CHECKING | 1.25 | AMPS |
| 4 | BOARD CHECKING | YES / NO |  |
| 5 | MOTOR CHECKING | YES / NO |  |
| 6 | MACHINE LED FITTING | YES / NO |  |
| 7 | AIR LEAKAGE SAVING CFM | 0.5 | CFM |
| 8 | CLEANING WORK | YES / NO |  |
| 9 | REMARKS | VALUES OR NOS |  |
| LINE TABLE MAINTENANCE PREVENTIVE MAINTENANCE ENTRY |  |  |  |
| UNIT | GARMENTS 1&2 FLOOR | START TIME | 12:30 PM |
| LINE NO | 15 | END TIME | 1:30 PM |
|  |  | TOTAL TIME | 01:00 |
| SL.NO | CHECK POINTS | EXAMPLES | UOM |
| 1 | BUS BAR CHECKING | YES/NO |  |
| 2 | PLUG TOP CHECKING | YES/NO |  |
| 3 | AIR LEAKAGE SAVING CFM | 0.5 | CFM |
| 4 | TUBE LIGHT CHECKING | YES/NO |  |
| 5 | NUMBER OF MACHINE RUNNING | 15 |  |
| 6 | LOAD AMPS CHECKING | 0.5 | AMPS |
| 7 | CLEANING WORK | YES/NO |  |
| 8 | CONNECTOR CHECKING | YES/NO |  |
| 9 | REMARKS | VALUES OR NOS |  |

REMARKS: